## Dear Doctor and Team:

In order to optimize our patient’s esthetic and functional result, we would like your office to provide a surgical implant stent for each implant patient.  I have formulated these stent guidelines to help your laboratory with fabrication of the implant surgical guide.  Please schedule the patient for impressions in your office and have the lab deliver the stent directly to our office prior to the surgery date.  A surgical guide will help facilitate implant placement and optimize our outcome.

You may use the dental lab of your choice.  They will fabricate the stent according to our specifications, minimizing time and preparation in your office.

Dr. Derhalli’s stent protocol:

(please forward this information to your lab)

Please fabricate an acrylic implant surgical guide with the following specific criteria:

- Anterior stents should have the lingual aspect open with a groove/slot placed on the lingual/palatal aspect emerging at about the incisal edge, specifically aligned with the proposed implant site on the model.

- Posterior stents should have the occlusal access centered within the central groove, specifically aligned with the proposed implant site on the model.

- The lingual/palatal slot or the occlusal access should be large enough to fit a 3mm twist drill or a wooden Thompson Marker.)

- The surgical guides should extend at least 2-3 teeth on either side of the edentulous area to reinforce stabilization of the stent and minimize any movement.

Typical guidelines are as follows:

- Take Upper and Lower Impressions and Bite Registration

- Mark the impression to indicate implant placement for the final restorations(s)

- Call Lab for a pick-up--Allow 5 days at Lab

- On lab slip indicate “Per Dr. Derhalli’s Implant Stent Protocol”

- Lab will deliver stent to our office

If we can be of further assistance, please contact our office at 360-254-8151.

Thank you,

Dr. Derhalli and Team